DECON Reeves Tent Setup

- Lay out tarp to protect tent.

- Lay out the berm (black tarp looking)

  - Sides – Collapsed and flipped under

  - Entrance/Exit – Collapsed and flipped forward

  - Yellow safety markings facing upward

- Put tent in middle of berm

- Remove 4 white tent poles

- Wire loops in row indicate side walls

- Loosen straps

- Have one person at each corner

- Lift and expand shelter by walking backwards

- Flip doors and side fabric over so they are hanging down

- 2 red tab lifting points are located on each end

- Lifting together, place white poles under red tabs for support (they do not insert into holes)

- Center shower wands, shower heads and lights can be installed:

  - The 2 red ones are inject solution lines

  - The 2 black ones are rinse lines

  - These are installed for non-ambulatory patients.  Red is entrance, black is exit.

- Now lift shelter with poles until side walls are level on ground.  There is no click when extended up.

- Berm walls can now be set up around side walls up around.  Side walls up first, one entrance left down for setup.

Grids:

- Place down grids on both side of rollers and all flooring all around.

- Put up entrance berm

- Place submersible pump inside at lowest corner and attach discharge hose

- Install lane curtains and anchor cords to grid

DECON Tent Teardown

Let tent dry out before storing:

- Remove Curtains

- Remove wands and showerheads and replace with caps

- Remove discharge hose from pump and remove pump from berm

- Remove grids and rollers

- Collapse side walls under

- Collapse entrance/exit wall forward

- Lift corners of shelter and flap over berm so it can be collapsed

- Lift side walls over berm

- Locate striking positions (2 each side).  Place hand under flap

- SAFETY: No one inside can be inside and ensure before collapse

- Lift and step back to strike out for collapse

- Lift shelter and walk inward

- Place straps (2) around and cinch up

- Place on side and re-cinch

- Put bag over shelter after standing shelter up

- Lay down shelter and put poles in bag

- Cinch tie on top of box

- Tighten outside straps

- Fold berm length-wise in quarters.

Collapsible Rollers:

- Expand and center

- Adjust height, locate second locking, slide up ring, turn knob inside to tighten.

- Turn wheels inside.